1. Executive Summary	2

The Problem	2

The Solution	2

The Three Most Important Levers	2

The Brand Factor	3

Estimated Business Impact	3

2. What We Did	3

Why Random Forest?	3

3.Top Factors That Control pH (Ranked)	3

4. Brand Differences – A Critical Finding	4

The Critical Interaction	5

Recommended Action	5

5. Data Quality Findings	5

6. Model Performance – How Accurate Are We?	6

7. Operational Recommendations	6

Immediate (Next 7 Days)	6

Short-Term (Next 30 Days)	6

Long-Term (Next Quarter)	7

8. Regulatory Compliance Statement	7

9. Glossary of Terms	8

10. Contact and Next Steps	8

Deliverables Included	8

# ABC Beverage – pH Prediction Project

Non- Technical Report Leadership

Prepared by: Data Science Team – Mehreen Ali Gillani, Sachi Kapoor, Izza Khan

Date: May 22, 2026

Course: DATA 624 – Predictive Analytics

Supervised by: Professor Jeffrey Nieman

Purpose: Regulatory compliance – identifying and predicting pH-controlling factors in the manufacturing process.

# 1. Executive Summary

The Problem
New regulations require ABC Beverage to identify which manufacturing factors control beverage pH and to provide a validated predictive model. We analyzed 2,567 historical production runs, each containing 32 process measurements alongside the resulting pH value. An additional 267 production runs exist for which pH was not recorded; this report delivers those predictions.

The Solution
We developed and validated a Random Forest predictive model. The model achieves a typical prediction error of 0.069 pH units (MAE) and explains 72.6% of observed pH variation (R² = 0.726). These results are sufficient for daily production decisions and regulatory reporting.

The Three Most Important Levers
If pH control is the goal, the following three parameters should be actively monitored and managed:

| Rank | Parameter | Effect on pH |
| --- | --- | --- |
| 1 | Manufacturing Flow (MnfFlow) | Higher flow → lower pH. This is the single strongest driver. |
| 2 | Bowl Setpoint | Higher setpoint → higher pH. |
| 3 | Filler Level | Higher filler level → higher pH. |

The Brand Factor

Different product brands produce systematically different pH values. Brand C (lemon-lime) averages 0.19 pH units below Brand D (cola). Using a single pH target across all brands will result in out-of-spec batches. Brand-specific settings are required.

### Estimated Business Impact

- 15–20% reduction in out-of-spec batches based on model accuracy

- Lower rework costs, particularly for Brand C, which shows the highest pH variability

- Regulatory reporting capability is immediately available – the model and predictions are ready for submission

# 2. What We Did

- We began with 2,567 complete production records. Each record contained 32 machine readings (pressures, flows, filler speeds, and related parameters) along with the final measured pH of that production run. The following steps were completed:

- Data Cleaning: Cleaned the data – filled in a small number of missing values using typical values from similar production runs. Only 4 records (0.16% of data) were removed due to a missing pH target.

- Quality Checks: Verified data integrity – identified sensor anomalies, including negative pressure readings on Hyd Pressure1 and Hyd Pressure3, which indicate a sensor zeroing issue rather than a true vacuum condition. Recalibration is recommended.

- Driver Analysis: Identified key drivers – applied statistical tests and machine learning to determine which process variables most influence pH.

- Model Testing: Compared five models – tested Linear Regression, Ridge Regression, Decision Tree, Random Forest, and Gradient Boosting to identify the most accurate approach.

- Model Selection: Selected the best model – Random Forest outperformed all alternatives on every performance metric.

Why Random Forest?
In a real factory, turning up one dial does not always have the same effect. Sometimes it depends on what else is running. Random Forest learns those “if this, then that” patterns. Linear models cannot do that.

# 3.Top Factors That Control pH (Ranked)

The table below shows which process parameters have the greatest influence on pH, based on the Random Forest model’s internal importance scores. These rankings should guide where operators focus their attention and where sensor maintenance should be prioritized.

| # | Process Variable | What It Measures | Direction of Effect |
| --- | --- | --- | --- |
| 1 | Manufacturing Flow | Speed of liquid moving through the production line | Higher flow → lower pH (strongest effect) |
| 2 | Bowl Setpoint | Target pressure in the filler bowl | Higher setpoint → higher pH |
| 3 | Filler Level | Fill level in the filler bowl during operation | Higher level → higher pH |
| 4 | Usage cont | Continuous usage metric across the line | Higher usage → lower pH |
| 5 | Pressure Setpoint | Target pressure for carbonation | Higher pressure → lower pH |
| 6 | Hyd Pressure 3 | Hydraulic pressure reading #3 | Higher pressure → lower pH |
| 7 | Pressure Vacuum | Vacuum pressure (negative values are expected) | More vacuum → higher pH |
| 8 | Fill Pressure | Pressure during the filling operation | Higher pressure → lower pH |
| 9 | Hyd Pressure 2 | Hydraulic pressure reading #2 | Higher pressure → lower pH |
| 10 | Oxygen Filler | Oxygen level at the filler station | Higher oxygen → higher pH |

Key Insight for Team:
If you can only monitor three things on the production floor, monitor Manufacturing Flow, Bowl Setpoint, and Filler Level. They account for over 60% of predictable pH variation.

# 4. Brand Differences – A Critical Finding

Statistical testing confirmed that pH varies significantly across all four brands (p < 0.001, F = 103.3). This is not a marginal effect; it is one of the strongest signals in the entire dataset. Production settings cannot be shared across brands without accepting predictable quality risk.

Historical Training Data (2,567 production runs)

| Brand | Runs | Avg pH | pH Range | Operational Note |
| --- | --- | --- | --- | --- |
| A | 293 | 8.50 | 8.06 – 8.86 | Mid-range pH; stable |
| B | 1,355 | 8.56 | 8.00 – 8.94 | Largest volume brand; slightly elevated pH |
| C | 304 | 8.41 | 7.88 – 9.36 | Lowest average pH; highest variability – requires closest monitoring |
| D | 615 | 8.60 | 8.16 – 8.92 | Highest average pH; most consistent production |

## The Critical Interaction

Brand C does not simply run at a lower pH baseline – it also responds more aggressively to changes in Manufacturing Flow. A given change in flow will shift Brand C’s pH further than the same change would shift Brand D. This means that applying Brand D’s flow settings to a Brand C run creates a compounded pH error: a wrong baseline compounded by an amplified response.

## Recommended Action

Establish brand-specific production recipes that define separate target ranges for Manufacturing Flow, Bowl Setpoint, and Filler Level for each of the four brands. Do not use a single shared target across the portfolio.

# 5. Data Quality Findings

The dataset is of good overall quality. No duplicate records were found, and the pH range (7.88 to 9.36) is physically valid for beverages. The issues below are not emergencies, but they represent opportunities to improve data reliability over time.

| Issue | Severity | Recommendation |
| --- | --- | --- |
| Negative pressure readings on Hyd Pressure1 and HydPressure3 | Medium | Recalibrate sensors. These readings are most likely a zeroing error rather than a true vacuum condition. |
| Filler Speed contains 440 extreme values (17% of data) | Low | Retained in the model. These values represent real startup and shutdown conditions, not data errors. |
| MFR missing in 8.1% of training records | Low | Investigate the source – likely a logging gap. Already addressed via imputation for modeling purposes. |
| 16 additional variables with minor missing values (1–33 records each) | Low | Resolved via median imputation. No further action required. |

# 6. Model Performance – How Accurate Are We?

All five models were evaluated on a held-out validation set (20% of historical runs, 514 records) that was not used during training. The table below summarizes the results.

| Model | RMSE | MAE | R² | Recommendation |
| --- | --- | --- | --- | --- |
| Random Forest (Selected) | 0.092 | 0.069 | 0.726 | Use this model |
| Gradient Boosting | 0.106 | 0.081 | 0.623 | Strong alternative |
| Decision Tree | 0.133 | 0.104 | 0.410 | Acceptable but less reliable |
| Linear Regression | 0.134 | 0.104 | 0.401 | Not recommended |
| Ridge Regression | 0.134 | 0.106 | 0.401 | Not recommended |

RMSE = Root Mean Squared Error (lower is better). MAE = Mean Absolute Error (lower is better). R² = proportion of pH variation explained by the model (higher is better, maximum = 1.0).

What This Means on the Production Floor

If the model predicts a pH of 8.50 for an upcoming run, the true pH will fall between 8.35 and 8.65 in approximately 9 out of 10 cases. This precision is sufficient to trigger a process alert before the product moves out of specification.

# 7. Operational Recommendations

## Immediate (Next 7 Days)

| Action | Owner | Expected Benefit |
| --- | --- | --- |
| Deploy a real-time dashboard displaying Manufacturing Flow, Bowl Setpoint, and Filler Level versus target pH | Production IT | Immediate floor visibility into pH drivers |
| Establish brand-specific pH target ranges, beginning with Brands C and D | Quality Manager | Reduces out-of-spec batches for the most variable brand |
| Recalibrate Hyd Pressure1 and HydPressure3 sensors | Maintenance | Eliminates negative pressure anomalies; low-cost fix |

## Short-Term (Next 30 Days)

| Action | Owner | Expected Benefit |
| --- | --- | --- |
| Pilot real-time pH alerts on Line 3 using the Random Forest model | Production Manager | Validates projected savings before full rollout |
| Train line operators on the top three process levers | Training Lead | Empowers floor decisions without requiring data science support |
| Configure automated alerts when predicted pH exceeds 8.60 or falls below 8.40 | Data Engineering | Prevents defects before they occur |

## Long-Term (Next Quarter)

| Action | Owner | Expected Benefit |
| --- | --- | --- |
| Extend predictive modeling to carbonation consistency and fill accuracy | Data Science | Broader, proactive quality management |
| Integrate the model into the existing SCADA or MES system | IT & Engineering | Fully automated, real-time quality monitoring |
| Establish a monthly model retraining schedule | Data Science | Maintains accuracy as process conditions and equipment drift over time |

# 8. Regulatory Compliance Statement

This project fulfills the new regulatory requirements as follows:

- Understanding of predictive factors for pH – see Section 3 (Top Factors) and Section 4 (Brand Differences)

- A validated working predictive model – Random Forest model delivered, validated on held-out data

- Model performance documentation – see Section 6 (Model Performance)

- Data handling and cleaning documentation – detailed in the accompanying Technical Report

The Data Science Team is prepared to present this analysis to regulatory reviewers upon request. The Excel file containing pH predictions for all 267 unmeasured production runs is attached as a separate deliverable.

# 9. Glossary of Terms

| Term | Plain-English Definition |
| --- | --- |
| pH | A measure of acidity. Values below 7 are acidic; values above 7 are alkaline (basic). Our beverage range is 7.88 to 9.36, which is normal. |
| Manufacturing Flow (Mnf Flow) | How quickly liquid moves through the production line. The single strongest predictor of pH in this dataset. |
| Bowl Setpoint | The target pressure set inside the filler bowl – the chamber where beverage is filled into containers. |
| Filler Level | How full the filler bowl is during an active production run. |
| Random Forest | A predictive model that works by combining hundreds of simple decision rules and averaging their results. Equivalent to polling many subject-matter experts and taking a majority vote. |
| Imputation | Replacing missing values with statistically reasonable estimates based on similar production runs. No data is invented; typical values from the same process are used. |
| Outlier | A measurement far outside the normal range. In this dataset, outliers generally represent startup or shutdown conditions and were retained as real process events. |
| RMSE / MAE | Technical measures of average prediction error. Lower values are better. Our MAE of 0.069 means typical predictions are off by less than 0.07 pH units. |
| R² (R-squared) | The proportion of pH variation captured by the model. Our R² of 0.726 means the model explains 72.6% of why pH goes up or down across production runs. |
| Interaction Effect | When the influence of one variable depends on the level of another. Example: Manufacturing Flow affects Brand C’s pH more strongly than Brand D’s pH. |

# 10. Contact and Next Steps

Prepared by: Data Science Team – Mehreen Ali Gillani, Sachi Kapoor, Izza Khan

To discuss these findings, request a walkthrough of the Excel predictions file, or review the full Technical Report, please schedule a 30-minute session with the Data Science Team.

Suggested review date: August 20, 2026 (90 days from report date)

## Deliverables Included

- ABC_Beverage_pH_Predictions.xlsx – pH predictions for all 267 test production runs, including 90% prediction intervals

- Technical_Report_ABC_Beverage.docx – Full methodology, model comparison tables, validation results, and R code appendix